Bit stream correlation:

Papers:

Chiani for robust frame synchronization and sync-marker 
Pfletschinger for packet-location methods 
The 2024 Sensors paper if you later want ML-assisted blind frame detection

1. Chiani & Martini (2006)

Paper: Marco Chiani and Maria G. Martini, “On Sequential Frame Synchronization in AWGN Channels,” IEEE Transactions on Communications, vol. 54, no. 2, pp. 339–348, 2006. DOI: 10.1109/TCOMM.2005.863727.

Problem addressed

The paper studies how to detect the position of a known synchronization word—also called a sync marker, start code, or preamble—when it is embedded at an unknown position in a received noisy bitstream.

The receiver slides a window equal to the sync-word length across the received stream. At each bit/symbol position, it decides between two hypotheses:

H0:the window contains ordinary data

H1:the window contains the known synchronization word

If the metric exceeds a threshold, the receiver declares that it has detected the frame start; otherwise, it shifts the window by one sample and repeats.

Core contribution

The authors derive:

- An optimal likelihood-ratio test (LRT) for frame synchronization in an AWGN channel when soft values and channel/SNR information are available.

- A generalized likelihood-ratio test (GLRT) for situations in which the data distribution is unknown.

- Analytical expressions for:

  - False-alarm probability, also called sync-word emulation probability.

  - Missed-detection probability.

  - Receiver-operating-characteristic curves.

- A comparison against conventional hard-decision and soft-decision correlation methods.citeseerx.ist.psu

The main conclusion is important: simple correlation is common, but it is not always the best metric—especially when the receiver has soft information. Their LRT and GLRT metrics can materially improve synchronization performance relative to standard correlation.citeseerx.ist.psu

Key result

The paper finds that:

- LRT is theoretically optimal when the channel statistics and data distribution are known.

- GLRT is simpler, does not require SNR knowledge or an assumed data distribution, and performs close to LRT except under very poor channel conditions.

- Ordinary soft correlation can sometimes be worse than hard correlation; soft values must be used with an appropriate likelihood-based metric, not automatically through a raw correlation sum.citeseerx.ist.psu

In one reported comparison, for a 32-bit sync word and a target detection performance, GLRT required about 2 dB less Eb/N0than soft-correlation synchronization under the tested setting.citeseerx.ist.psu

Relevance to your project

This is the most immediately useful paper for implementing your bitstream correlation module.

You can use it to implement:

- Sliding sync-word search.

- Hard Hamming-distance detection.

- Soft-bit correlation.

- Likelihood-ratio-based sync detection.

- GLRT-based detection when SNR and data statistics are unknown.

- Threshold selection based on false-alarm tolerance.

- ROC evaluation for your bitstream-analysis GUI.

For an initial implementation, you can support known/common sync words such as:

text

0x1ACFFC1D   CCSDS transfer-frame sync marker

0xD3917E42   Common telemetry-related marker

0xEB90       Common short sync word

0xAA55       Common alternating/header pattern

For each candidate offset k, calculate a score. For hard bits, a simple baseline is normalized bipolar correlation:

Ck=i=0L-1(2rk+i-12si-1

where ris the received hard-bit sequence, sis the candidate sync word, and Lis the sync-word length.

A robust GUI output could display:

| Field | Example |
| --- | --- |
| Sync candidate | 0x1ACFFC1D |
| Best bit offset | 18,432 |
| Correlation score | 0.94 |
| Hamming distance | 2 / 32 |
| Threshold | 0.85 |
| Detection result | Candidate frame start |
| Confidence | High |

Limitation: Chiani’s paper assumes the sync word itself is known. It does not solve fully blind recovery of an unknown sync word and unknown frame length.citeseerx.ist.psu

2. Pfletschinger (2026)

Paper: Stephan Pfletschinger, “Frame Synchronization for a Single Packet,” Journal on Wireless Communications and Networking, 2026, Article 61. DOI: 10.1186/s13638-026-02629-2.

Problem addressed

Traditional frame synchronization often assumes a continuing transmission in which a known sync word appears periodically. Pfletschinger instead studies a different problem:

A receiver has one received observation block that contains exactly one packet at an unknown location, possibly surrounded by noise.

The packet consists of:

- A known preamble or sync word.

- Unknown data.

- Noise-only samples before and after the packet.

The task is to identify the packet’s starting position.link.springer

Core contribution

The key idea is that synchronization should use the entire packet, not only the known preamble.

Instead of measuring only:

preamble correlation

the paper derives a maximum-likelihood metric that considers:

- The known sync-word samples.

- The statistical structure of the unknown payload.

- The fact that samples outside the packet should contain only noise.

For BPSK, the paper derives an ML metric combining:

ΛMLμ=n=0S-1snρμ+n+n=SS+D-1lncosh⁡ρμ+n

where:

- μis a possible packet-start location.

- Sis preamble length.

- Dis payload length.

- snis a known preamble symbol.

- ρnis a received-sample reliability value related to the noise level.

The first term is essentially the preamble-match term. The second term captures evidence that the rest of the hypothesized packet contains valid signal-like BPSK data rather than only noise.link.springer

Simplified maximum-likelihood metric

The exact ML metric can be complex and depends on noise power. The paper derives a simpler near-optimal approximation using:

lncosh⁡x≈∣x∣-ln2

This produces a simplified ML score that combines preamble correlation with an energy/reliability measure over the payload region.

The useful result is that the simplified method:

- Performs nearly as well as the full ML metric in the paper’s simulations.

- Has only slightly more complexity than correlation.

- Does not require exact prior knowledge of noise power.

- Outperforms correlation-only detection.link.springer

Modulations covered

The paper derives methods for:

- BPSK in AWGN.

- OOK in an incoherent receiver setting.

- BPSK with Manchester coding.

- OOK with Manchester coding.

It also discusses a practical single-packet scenario such as ADS-B-like burst transmissions.link.springer

Key result

The paper reports that whole-packet ML and simplified-ML synchronization outperform:

- Ordinary preamble correlation.

- Metrics designed for periodically repeated sync words.

- A correlation-plus-energy heuristic.

In the BPSK and OOK cases studied, the simplified ML metric closely approaches the optimum full-packet ML performance. The article also reports cases where reliable synchronization is possible even without a conventional sync word, provided that usable packet/data structure is present.link.springer

Relevance to your project

This paper is useful when your input IQ/WAV file contains bursts, short transmissions, or one packet in a longer capture.

For your GUI pipeline, use it after modulation classification and demodulation:

text

IQ / WAV capture

→ Signal detection

→ Burst segmentation

→ Modulation classification

→ Soft-symbol or soft-bit extraction

→ Preamble correlation

→ Whole-packet score

→ Select best packet start

→ Demodulate payload

→ CRC/FEC validation

A practical simplified score for your first implementation can be:

Scorek=w1⋅PreambleCorrelationk+w2⋅PayloadEnergyk-w3⋅OutsidePacketEnergyk

This is not the full paper’s ML detector, but it provides a practical bridge from simple correlation to structure-aware burst detection.

Limitation: the method assumes packet length and modulation/data model are known or at least hypothesized. It is not, by itself, a fully blind method for unknown protocols, unknown packet lengths, unknown coding, and unknown preambles.link.springer

3. Wei et al. (2024), Sensors

Paper: Jiazheng Wei, Shitian Zhang, Mingchao Jin, Xiandeng He, Dongxiao Quan, and Chen Chen, “Blind Recognition of Frame Synchronization Based on Deep Learning,” Sensors, vol. 24, no. 20, article 6767, 2024. DOI: 10.3390/s24206767.

Problem addressed

This paper targets the hardest of the three scenarios: non-cooperative blind frame recognition.

In this setting, the receiver may not know:

- The frame length.

- The sync-code value.

- The exact location of frame boundaries.

- The original communication protocol.

- The bitstream’s error rate.

This is more closely related to your proposed bitstream-forensics stage than ordinary known-sync-word correlation.mdpi

Core idea

Instead of applying autocorrelation or pattern matching directly to the bitstream, the authors convert binary data into images.

Their workflow is:

- Take a received binary sequence.

- Apply flat-top interpolation to make structural features visually more prominent.

- Reshape the bit sequence into matrices with different candidate column counts.

- Convert the matrices into grayscale images.

- Add a scaling factor and create RGB images.

- Train a ResNet-50 CNN to classify:

  - Frame length.

  - Sync-word category.

  - Or joint frame length plus sync-word category.mdpi

Why the image conversion works

When the matrix column count equals—or is close to—the true frame length, periodically repeating synchronization bits align approximately into vertical or slanted stripes.

- If the selected matrix width equals the actual frame length, the sync pattern tends to form vertical stripe-like structures.

- If the width is slightly smaller or larger, the stripe becomes diagonal.

- If the width is far from the true frame length, the pattern appears random and stripe structure disappears.

The CNN learns these stripe patterns to infer frame parameters.mdpi

Reported results

The authors report simulation-based results for selected frame lengths and predefined synchronization-code classes:

- 100% frame-length recognition when BER is below 0.20.

- More than 90% recognition when BER reaches 0.25.

- More than 95% sync-code recognition at BER 0.25 in one tested setup.

- Better results than several classical comparison methods under the authors’ chosen simulation conditions.mdpi

These numbers are promising, but they should be interpreted carefully: the model was trained and evaluated on constrained, predefined classes of frame lengths and sync words.

Critical limitation

The authors explicitly acknowledge that the approach is not fully general blind recognition.

The trained neural network can recognize only the limited set of frame-length and sync-word categories included in training. In other words:

- It cannot automatically identify any arbitrary unknown protocol.

- It may not generalize to signal formats not represented in the dataset.

- It has high compute cost: ResNet-50 has about 25.6 million parameters and roughly 4.1 billion floating-point operations, while classical approaches can have approximately linear complexity.mdpi

This limitation matters strongly for your project. You should not present it as a universal unknown-protocol solution unless you train and test it across realistic modulation, coding, noise, burst, and channel conditions.

Comparison

| Aspect | Chiani & Martini | Pfletschinger | Wei et al. |
| --- | --- | --- | --- |
| Main goal | Find a known sync word in a noisy stream | Locate one packet using preamble plus payload evidence | Blindly infer frame length and sync-word category |
| Sync word known? | Yes | Yes, generally | No, but categories must be in the training set |
| Frame length known? | Not necessarily for sequential search | Generally packet length is assumed/hypothesized | Estimated from learned classes |
| Input type | Hard bits or soft values | Received symbols/samples | Binary bitstream |
| Main technique | LRT, GLRT, correlation analysis | Maximum likelihood using entire packet | Bitstream-to-image transformation plus ResNet-50 |
| Best use in your project | Initial bitstream correlation and frame-start detection | Burst/single-packet localization | Research-level blind frame-structure hypothesis generator |
| Complexity | Low to moderate | Moderate | High |
| Validation approach | False-alarm and missed-detection ROC analysis | Frame synchronization error simulations | Classification accuracy over trained classes |

Recommended Implementation Path

For your current project, follow this progression:

- Implement Chiani-style sync detection first.
Add sliding hard correlation, soft correlation, Hamming distance, configurable thresholds, and known sync-word libraries. This is manageable, explainable, and directly useful in the GUI.citeseerx.ist.psu

- Add packet/burst segmentation and a Pfletschinger-inspired whole-packet score.
Use preamble correlation plus payload energy/reliability to identify the best packet position in a burst capture. Validate it using CRC once your CRC module is ready.link.springer

- Add ML only after you have data.
Generate or collect labelled bitstreams with known frame lengths, sync words, SNRs, BERs, modulation types, interleavers, and FEC schemes. Train an ML model only as a hypothesis-ranking component, then confirm decisions with deterministic checks.

De-interleaving:

Yes. Beyond the two papers already discussed—the IEEE Access matrix-multiplication method and Gan et al.’s convolutional-interleaver estimator—there are several highly relevant de-interleaving papers. The most useful additional ones for your project are:

- Burel & Gautier’s rank/Gauss–Jordan method — foundational for blind block-interleaver detection.

- Wee, Choi & Jeong’s Kolmogorov–Smirnov test method — a practical improvement for noisy and short data.

- Swaminathan et al.’s joint FEC/interleaver classification work — useful because real captures often have unknown coding and interleaving simultaneously.

- Ramabadran et al.’s RS-plus-interleaver reconstruction — especially relevant if you plan Reed–Solomon FEC.

- A repetitive-pattern method for convolutional interleavers — helpful where the channel code is a block code and the interleaver is convolutional.

The central lesson across these papers is that blind de-interleaving generally cannot be solved from “random-looking bits” alone. These methods exploit residual structure introduced by channel coding—especially linear dependencies in block codes or convolutional codes—to estimate an interleaver’s period, dimensions, alignment, and sometimes its permutation.ieeexplore.ieee+1

1. Burel & Gautier: Foundational Block-Interleaver Method

Paper: G. Burel and R. Gautier, “Blind Detection of Interleaver Parameters,” Signal Processing, vol. 89, no. 4, pp. 450–462, 2009.

Main problem

The paper considers a non-cooperative receiver that intercepts a coded and interleaved binary stream but does not know:

- The channel-code structure.

- The interleaver length.

- The interleaver starting position.

- The codeword alignment.

- The block-interleaver dimensions.

The objective is to estimate these parameters blindly before attempting de-interleaving and channel decoding.hal+1

Core idea

The method relies on the fact that a channel-coded bitstream is not completely random. Linear block codes create linear dependencies among bits in a codeword.

A block interleaver rearranges bits, but it does not eliminate those algebraic dependencies. If you guess the correct interleaver period and correctly reshape/de-interleave the stream, matrices built from received data show rank deficiency.

The procedure is conceptually:

- Choose a candidate interleaver period P.

- Choose a candidate start offset o.

- Reshape portions of the bitstream into matrices.

- Apply Gaussian or Gauss–Jordan elimination over GF2.

- Measure matrix-rank behavior.

- Compare rank behavior with what would be expected for random binary data.

- Select candidate periods and offsets with unusually strong rank deficiency.

- Use these candidates to infer interleaver dimensions and codeword alignment.

For truly random binary matrices, full rank is relatively common. For matrices derived from aligned coded data, parity relationships cause systematic rank reduction.hal+1

Why it matters

This paper is a foundation for many later methods, including the IEEE Access matrix-based method you mentioned. It provides the key principle behind blind block de-interleaving:

Correct alignment + correct candidate dimensions⇒detectable algebraic structure

Strengths

- Classical, interpretable, non-ML approach.

- Does not require a labelled training dataset.

- Uses the mathematical structure of channel coding.

- Appropriate for block interleavers associated with linear block codes.

- Useful for estimating interleaver period and synchronization/alignment.hal+1

Limitations

- Sensitive to bit errors; rank properties can degrade rapidly with noisy hard decisions.

- Needs enough captured bits to build meaningful matrices.

- Can be computationally expensive if you search too many candidate periods and offsets.

- May struggle when the code family itself is unknown.

- Less suited to encrypted/compressed data without identifiable FEC structure.

Use in your project

This is a strong candidate for your first serious blind block-interleaver estimator.

Add a module such as:

text

deinterleaving/

├── gf2_rank.py

├── rank_profile.py

├── period_search.py

├── alignment_search.py

├── block_hypotheses.py

└── block_deinterleaver.py

Your GUI can show a heatmap:

- X-axis: candidate interleaver period.

- Y-axis: candidate bit offset.

- Cell value: rank-deficiency score.

The correct or near-correct candidate should appear as a noticeable peak.

2. Wee, Choi & Jeong (2021): Kolmogorov–Smirnov Test

Paper: Seungwoo Wee, Chang Woo Choi, and Jae Hoon Jeong, “Blind Interleaver Parameters Estimation Using Kolmogorov–Smirnov Test,” Sensors, vol. 21, no. 10, article 3458, 2021. DOI: 10.3390/s21103458.

Main problem

This paper targets blind estimation of interleaver parameters under noise, especially where rank-based methods may become unreliable or require extensive computation.

The authors note that the rank distributions of matrices built from linear-code sequences differ from the rank distributions of matrices built from random sequences. They use a statistical test to quantify that difference.mdpi+1

Core idea

Instead of relying only on a single rank-deficiency value, the method:

- Assumes a candidate interleaver period.

- Rearranges the intercepted sequence into several square binary matrices.

- Computes the rank of each matrix over GF2.

- Builds an empirical rank distribution.

- Compares this distribution with a random-sequence rank distribution.

- Uses the Kolmogorov–Smirnov (K–S) test to measure the maximum difference between the two cumulative distributions.

- Uses the K–S statistic as a score for candidate interleaver periods.

The K–S statistic is:

D=supxFobservedx-Frandomx

where:

- Fobservedxis the cumulative distribution of ranks observed from the captured data.

- Frandomxis the expected cumulative rank distribution for random data.

- A larger Dindicates that the candidate data behaves less like random data and more like structured coded data.

The method also uses multinomial-distribution analysis to control false-alarm probability.mdpi

Key contribution

The authors use the K–S value as a robust decision score rather than merely checking whether one matrix has a reduced rank.

Their reported experiments indicate improved estimation performance relative to previous methods across different bit-error-rate conditions.mdpi

Strengths

- Better statistical grounding than a single rank test.

- Explicit false-alarm control using probability modelling.

- Suitable for noisy received bitstreams.

- Useful for ranking candidate interleaver periods.

- Compatible with binary hard-decision streams.

- Does not require a neural-network training dataset.mdpi

Limitations

- It estimates interleaver period more directly than full arbitrary permutation recovery.

- Still assumes coded data has detectable linear-code structure.

- Requires selecting matrix dimensions and sufficient windows.

- Depends on accurate or simulated reference distributions for random matrices.

- Performance may degrade if demodulation errors are high or the code is nonlinear/unknown.

Use in your project

This is probably the best paper to implement after the basic rank-deficiency method.

A good practical strategy is:

- Use bitstream autocorrelation to get rough periodicity candidates.

- Use the K–S rank-distribution score to rank likely interleaver periods.

- For top candidates, test matrix row/column dimensions.

- De-interleave using row/column hypotheses.

- Validate the output through FEC syndrome checks, sync correlation, or CRC passes.

Your result table could look like:

| Candidate period | K–S score | Rank-deficiency score | Sync score | CRC-valid frames | Final rank |
| --- | --- | --- | --- | --- | --- |
| 64 | 0.11 | 0.20 | 0.34 | 0 | 4 |
| 128 | 0.63 | 0.77 | 0.89 | 11 | 1 |
| 256 | 0.41 | 0.45 | 0.52 | 0 | 2 |

Do not interpret a high K–S value alone as proof. Treat it as an evidence score and require downstream validation.

3. Swaminathan et al. (2017): Joint FEC Classification and Interleaver Estimation

Paper: R. Swaminathan, A. S. Madhukumar, G. Wang, and T. S. Kee, “Classification of Error Correcting Codes and Estimation of Interleaver Parameters in a Noisy Transmission Environment,” IEEE Transactions on Broadcasting, 2017. DOI: 10.1109/TBC.2017.2704436.

Main problem

This paper recognizes an important practical issue: you may not know whether an intercepted stream is:

- Uncoded.

- Block coded.

- Convolutionally coded.

- Coded and then interleaved.

Trying to estimate interleaver parameters without knowing the broad FEC family can waste time and create false detections.

The paper therefore proposes joint recognition of FEC-code type and interleaver parameters without prior transmitter information.ieeexplore.ieee+1

Core idea

The approach first classifies received data symbols into three broad classes:

- Uncoded.

- Block-coded.

- Convolutional-coded.

It then estimates interleaver parameters using techniques appropriate for the likely code family. The paper includes analytical and histogram-based approaches for setting thresholds in noisy conditions.ieeexplore.ieee+1

For block-coded streams, it relies on properties related to linear dependency and rank behavior. For convolutional codes, it uses structural behavior appropriate to sequential coding rather than assuming independent block codewords.

Why it matters

This is highly useful for your system design because your input stream is unknown. Before running expensive block-interleaver searches, your pipeline should estimate:

Puncoded, Pblock-coded, Pconvolutional-coded

Then select the proper analysis branch:

text

Demodulated bits

│

▼

FEC-family classifier

│

┌──────┼─────────────────────┐

▼      ▼                     ▼

Uncoded Block-coded      Convolutional-coded

│       │                     │

│       ▼                     ▼

│   Block-interleaver     Convolutional-interleaver

│   period/dimension      depth/register search

│   estimation            and reconstruction

▼

Frame/sync/CRC analysis

Strengths

- Solves a more realistic joint problem.

- Prevents applying the wrong de-interleaver model.

- Includes noisy-channel thresholding considerations.

- Useful as a high-level orchestration paper for your architecture.ieeexplore.ieee+1

Limitations

- Broad classification is not equivalent to complete FEC reconstruction.

- A classifier can make an early wrong decision, causing downstream errors.

- In your implementation, retain several ranked hypotheses rather than using a rigid single branch.

- It should be followed by actual validation through CRC, syndrome, re-encoding, or repeated frames.

Use in your project

Implement a lightweight FEC family classifier before detailed de-interleaving. Its output should be probabilistic:

| FEC class | Confidence | Action |
| --- | --- | --- |
| Block code | 0.68 | Run block-interleaver period search |
| Convolutional code | 0.25 | Run limited convolutional-interleaver search |
| Uncoded | 0.07 | Run direct framing/sync search |

This is better than immediately testing every possible interleaver type.

4. Ramabadran et al. (2018): Reed–Solomon Plus Interleaver Reconstruction

Paper: R. Swaminathan Ramabadran, A. S. Madhukumar, G. Wang, and T. S. Kee, “Blind Reconstruction of Reed–Solomon Encoder and Interleavers Over Noisy Environment,” IEEE Transactions on Broadcasting, vol. 64, no. 4, pp. 830–845, 2018. DOI: 10.1109/TBC.2018.2795461.

Main problem

The paper addresses a realistic coded transmission chain:

Payload→Reed–Solomon encoder→Interleaver→Noisy channel

The receiver does not know the RS encoder parameters or the interleaver parameters, and it must reconstruct them from noisy intercepted data.ntu.edu

Core idea

The authors use the algebraic structure of Reed–Solomon codewords and their behavior in a finite field GF2m.

The receiver examines candidate codeword lengths, symbol alignments, and interleaver hypotheses. It searches for structures consistent with RS coding, then estimates:

- Codeword length.

- Message length.

- Field-related parameters.

- Symbol alignment.

- Interleaver configuration.

The method leverages both RS-code constraints and interleaver structure instead of treating FEC and interleaving as separate unrelated problems.ntu.edu

Why it matters

This is directly applicable to your intended L8–L9 design:

text

Demodulated bits

→ Byte alignment hypotheses

→ RS parameter hypotheses

→ Interleaver hypotheses

→ RS consistency / decoding evidence

→ De-interleaved, decoded output

→ CRC or header validation

The key design lesson is that the FEC decoder helps identify the interleaver, and the correct de-interleaver helps reveal the FEC structure. They should be treated as a joint hypothesis search.

Strengths

- Strong match for RS-coded telemetry or satellite-style links.

- Handles noise explicitly.

- Jointly considers FEC and interleaver reconstruction.

- More realistic than treating RS decoding and interleaving independently.ntu.edu

Limitations

- More complex than a simple block-interleaver search.

- Requires byte/symbol alignment hypotheses.

- Assumes the underlying code family is Reed–Solomon.

- Can require substantial data and careful finite-field implementation.

- Not a suitable first de-interleaving feature for a hackathon MVP.

Use in your project

Use this as a later advanced FEC-aware de-interleaving mode:

- First support user-selected RS configurations.

- Then automatically test a limited library of likely RS candidates.

- Test all eight bit offsets.

- Test MSB-first and LSB-first byte order.

- Test a limited set of block-interleaver dimensions.

- Score candidates by RS decoder success, corrected-symbol count, CRC validity, and repeated frame structure.

5. Convolutional-Interleaver Estimation Through Repetitive Patterns

Paper: “Estimation of Convolutional Interleaver in a Non-Cooperative Context,” ICACT conference paper.

Main problem

This work estimates unknown convolutional-interleaver parameters in a non-cooperative setting, particularly when the transmitted data was first protected by a block channel code.

The unknowns include:

- Interleaving depth.

- Shift-register size.

- Number of branches or delay lines.

- Codeword length needed to reconstruct the de-interleaver.icact

Core idea

A convolutional interleaver does not permute one finite block all at once. It uses multiple shift registers with different delays. This creates structured temporal displacement.

The method analyzes repetitive patterns in interleaved sequences. When input data has block-code structure, patterns and linear relationships recur in ways related to the interleaver’s delays. By detecting those repetitions, the receiver can estimate convolutional-interleaver parameters and reconstruct a compatible de-interleaver.icact

Strengths

- More directly relevant to convolutional interleavers than block-matrix methods.

- Uses code-induced repetition and structural patterns.

- Targets the non-cooperative scenario relevant to your problem statement.

- Can guide a staged parameter search instead of brute-forcing every delay-line configuration.icact

Limitations

- Relies on detectable repeated structure from the source channel code.

- May require longer captures than block-interleaver estimation.

- Sensitive to high BER and poor demodulation.

- The parameter search remains expensive if register count/depth bounds are wide.

- Less appropriate when the stream is encrypted, compressed, or lacks recognizable FEC structure.

Use in your project

For a basic implementation, limit the parameter range:

| Parameter | Initial candidate range |
| --- | --- |
| Number of branches | 2 to 16 |
| Register delay increment | 1 to 16 symbols |
| Interleaver depth | 2 to 16 |
| Candidate codeword length | 32 to 1024 bits |
| Input bit alignment | 0 to 7 |
| Bit order | MSB-first and LSB-first |

Then score every reconstructed output by:

S=w1Srank+w2Ssync+w3SFEC+w4SCRC+w5Speriodicity

A candidate must not be accepted based only on pattern repetition. It should ideally produce a valid FEC syndrome, a valid CRC, repeated headers, or a recognized frame structure.

6. Comparison of De-Interleaving Papers

| Paper / method | Interleaver type | Main technique | Needs FEC structure? | Noise robustness | Best role in your project |
| --- | --- | --- | --- | --- | --- |
| Burel & Gautier, 2009 | Block | GF2rank deficiency and Gauss–Jordan elimination | Yes, typically linear-code structure | Moderate | Foundational block-period and alignment search |
| IEEE Access, 2021 | Block | Matrix multiplication / rank-structure analysis | Yes | Designed for limited data | Practical block-interleaver estimator |
| Wee et al., 2021 | Primarily block-period estimation | K–S test on rank distributions | Yes | Good relative robustness under BER | Rank and prioritize candidate periods |
| Swaminathan et al., 2017 | Block and convolutional contexts | Joint FEC-family classification and parameter estimation | Yes | Explicit noisy-channel analysis | Select the right FEC/interleaver branch |
| Gan et al., 2013 | Convolutional | Estimate depth-width product, then reconstruct/test | Typically yes | Moderate | Main convolutional-interleaver implementation reference |
| ICACT repetitive-pattern method | Convolutional | Repetition-pattern analysis and reconstruction | Yes, especially block-coded input | Moderate | Secondary convolutional-interleaver strategy |
| Ramabadran et al., 2018 | RS-associated interleavers | Joint RS-code and interleaver reconstruction | Yes, specifically RS structure | Designed for noisy data | Advanced RS + de-interleaver stage |

Practical Implementation Priority

For your project, I recommend this order.

Phase 1: Bitstream Preparation

Before any de-interleaving:

- Produce hard bits and preferably soft LLRs.

- Test phase ambiguity.

- Test polarity inversion.

- Test differential decoding.

- Test MSB-first and LSB-first bit order.

- Search all eight byte-alignment offsets.

- Remove obvious scramblers where a known scrambler family is suspected.

- Segment bursts and estimate likely frame periodicity.

Without these steps, a valid interleaver algorithm will often fail because the bitstream is incorrectly aligned.

Phase 2: Block Interleaving MVP

Implement:

- Candidate period search.

- GF2matrix-rank computation.

- Rank-deficiency score.

- K–S rank-distribution score.

- Search over candidate row/column dimensions.

- Standard rectangular block de-interleaving:

  - Write by rows, read by columns.

  - Write by columns, read by rows.

- Scoring using sync correlation, CRC, and FEC checks.

Use Burel–Gautier plus Wee et al. as the main references.hal+1

Phase 3: FEC-Aware Validation

Add:

- Convolutional-code recognition.

- Rate-1/2, K=7Viterbi decoding.

- Common generator-polynomial testing.

- RS byte alignment and selected RS configurations.

- Syndrome/decoder-success score.

- Re-encoding or parity-check validation.

Use the 2017 joint FEC/interleaver-classification paper and the 2018 RS/interleaver paper as architecture references.ieeexplore.ieee+1

Phase 4: Convolutional Interleavers

Implement after block interleaving works:

- Candidate shift-register branch counts.

- Candidate delay increments.

- Depth/width-product pre-estimation.

- Reconstructed de-interleaver simulation.

- FEC-aware validation.

- Rank candidates rather than returning one unconditional answer.

Use Gan et al. as the primary algorithmic reference and the repetitive-pattern ICACT work as a secondary source.link.springer+1

Important Design Principle

Your GUI should not show:

text

Interleaver: Block, 16 × 32

unless there is strong validation.

It should show something like:

text

Best hypothesis:

Type: Rectangular block interleaver

Dimensions: 16 × 32

Input orientation: Row write → column read

Bit offset: 5

Evidence:

K–S rank score: 0.71

Rank-deficiency score: 0.83

Sync correlation: 0.94

Viterbi valid path: Yes

CRC-16 valid frames: 18 / 20

Confidence: 0.96

This evidence-first design matches your project’s original architecture and avoids falsely claiming successful blind de-interleaving when the output only appears statistically plausible.

FEC:

Yes. There are several additional papers worth using for the FEC-identification and reconstruction part of your project, beyond Marazin–Gautier–Burel for convolutional codes, Ramabadran et al. for RS plus interleavers, Xia & Wu for LDPC, and the 2023 cascade-network paper.

The most useful additions fall into five groups:

- Joint FEC-family classification — decide whether a stream is uncoded, block-coded, convolutionally coded, etc.

- Convolutional-code reconstruction — identify rate, constraint length, parity-check/generator relationships before Viterbi decoding.

- Reed–Solomon/BCH reconstruction — infer non-binary cyclic-code parameters from noisy byte streams.

- LDPC parity-check identification and synchronization — recover or test parity structure before decoding.

- Turbo/polar/ML approaches — later-stage extensions, not MVP features.

For your SIH project, start with convolutional codes and Reed–Solomon, then implement a known-H-matrix LDPC decoder. Fully blind LDPC, turbo, BCH, and polar-code recognition should be shown as future work unless you have substantial labelled test data and time.

1. Joint FEC-Type Classification

Swaminathan et al. (2017)

Paper: R. Swaminathan, A. S. Madhukumar, G. Wang, and T. S. Kee, “Classification of Error Correcting Codes and Estimation of Interleaver Parameters in a Noisy Transmission Environment,” IEEE Transactions on Broadcasting, 2017. DOI: 10.1109/TBC.2017.2704436.

What it solves

This paper deals with a realistic non-cooperative problem: the receiver does not know whether the intercepted bitstream is:

- Uncoded.

- Block coded.

- Convolutionally coded.

- Coded and interleaved.

Instead of assuming the FEC family beforehand, it jointly classifies the code family and estimates interleaver parameters in noisy conditions.ieeexplore.ieee+1

Main approach

The system first extracts code-related statistical and algebraic characteristics from the bitstream, then distinguishes broad FEC categories. It uses the likely FEC family to choose the appropriate interleaver-parameter estimation path.

Conceptually:

text

Demodulated bitstream

↓

FEC-family classifier

↓

Uncoded / block-coded / convolutional-coded

↓

Appropriate de-interleaver and decoder hypothesis search

Why it matters

This is a highly useful pipeline-design reference. It prevents your application from blindly trying every FEC decoder and every interleaver structure on every capture.

Your GUI should show ranked candidates, for example:

| Candidate FEC family | Confidence | Next action |
| --- | --- | --- |
| Convolutional code | 0.72 | Run rate/polynomial/Viterbi hypotheses |
| Linear block code | 0.21 | Run block-interleaver and parity/rank analysis |
| Uncoded | 0.07 | Run direct frame/sync/CRC analysis |

Limitation

This paper helps identify the likely class of coding; it does not automatically guarantee complete recovery of every generator polynomial, parity-check matrix, puncturing pattern, or interleaver permutation. Treat the output as a hypothesis generator and validate with CRC, syndrome checks, or re-encoding.ieeexplore.ieee

2. Convolutional-Code Reconstruction

Marazin, Gautier & Burel (2011)

Paper: M. Marazin, R. Gautier, and G. Burel, “Blind Recovery of k/nRate Convolutional Encoders in a Noisy Environment,” EURASIP Journal on Wireless Communications and Networking, 2011.

What it solves

This is one of the key papers for your initial FEC implementation. It aims to recover unknown convolutional encoder parameters from noisy intercepted data, including:

- Encoder rate k/n.

- Constraint length.

- Parity-check relationships.

- Generator matrix or generator polynomials.

- Encoder structure needed to construct a decoder.jwcn-eurasipjournals.springeropen

Main approach

The paper exploits the linear relationships introduced by convolutional coding. Rather than treating the received sequence as random, it searches for parity-check relations that remain detectable even after transmission errors.

The broad process is:

- Segment the recovered bitstream under candidate rates.

- Search for low-weight parity-check relationships.

- Infer a parity-check matrix.

- Derive candidate generator polynomials.

- Validate through re-encoding or decoding consistency.

- Build the corresponding Viterbi trellis only after the encoder parameters are credible.

Why it matters

This gives your project a defensible order:

text

Demodulate

→ Correct bit order / polarity / differential encoding

→ Estimate convolutional-code parameters

→ Build candidate trellis

→ Soft-decision Viterbi decode

→ Validate through CRC, frame structure, or re-encoding

Limitation

The search becomes difficult when:

- BER is high.

- The capture is short.

- The code is punctured.

- The encoder is recursive/systematic.

- The stream is interleaved before capture.

- Frame boundaries or codeword alignment are unknown.

For an MVP, limit support to common configurations such as rate-1/2, K=7, with 17181338, then add punctured rates later.jwcn-eurasipjournals.springeropen

Soft-GJETP Method (2022)

Paper: “A Soft-GJETP Based Blind Identification Algorithm for Convolutional Encoder Parameters,” 2022.

What it solves

This paper improves blind convolutional-encoder identification by using soft information, rather than relying solely on hard 0/1 decisions.

If your PSK/QAM/FSK demodulator outputs LLRs, they retain confidence information:

LLRbi=logPbi=0∣yiPbi=1∣yi

Large positive or negative values mean high confidence; values near zero mean uncertain bits.

Main approach

The method applies a soft-information form of a generalized joint error test pattern strategy to identify convolutional-code parameters. Rather than treating all bit decisions equally, it uses bit reliability when testing candidate parity/generator structures.ieeexplore.ieee

Why it matters

This is particularly relevant because your README already proposes Max-Log LLR generation after demodulation. If you preserve those LLRs, you can use them both for:

- Soft-decision Viterbi decoding.

- More robust blind identification of convolutional encoder parameters.

- Better candidate scoring under low SNR.

Limitation

This is more complex than a hard-decision implementation. Do not implement it before a conventional Viterbi decoder and basic hard-decision parameter search work correctly.

3. Reed–Solomon Reconstruction

Liu, Pan & Lei (2019)

Paper: P. Liu, Z. Pan, and J. Lei, “Parameter Identification of Reed–Solomon Codes Based on Probability Statistics and Galois Field Fourier Transform,” IEEE Access, vol. 7, pp. 33619–33630, 2019.

What it solves

The paper targets blind recognition of Reed–Solomon code parameters from received data, including noisy conditions. It focuses on identifying parameters such as:

- RS codeword length n.

- Primitive polynomial.

- Generator polynomial.

- Other structure needed to reconstruct the RS encoder/decoder.researching+1

Main approach

It combines:

- Probability statistics to discriminate plausible code parameters.

- Galois Field Fourier Transform (GFFT) to expose algebraic properties of Reed–Solomon codewords.

- A method designed to tolerate bit errors better than simple noiseless algebraic tests.

Reed–Solomon codes operate over GF2m, not individual bits. After converting candidate bitstreams into symbols, a valid RS codeword has roots related to its generator polynomial. The GFFT helps reveal these roots and thereby identify the code structure.researching+1

Why it matters

This is a strong reference for a later-stage RS-identification module:

text

Recovered bits

→ Test bit alignment 0…7

→ Test MSB-first / LSB-first

→ Form candidate \(GF(2^8)\) byte symbols

→ Estimate candidate RS codeword length

→ GFFT / statistical tests

→ Infer candidate primitive and generator polynomial

→ RS decode

→ Validate by decoder success, CRC, or repeated frame structure

Key caution

The paper is mathematically valuable, but it is not a simple plug-in decoder. You need:

- Correct byte alignment.

- Correct symbol order.

- Candidate field representation.

- Enough captured codewords.

- A strategy for interleaving, shortening, and puncturing.

For your project, implement known RS hypotheses first—such as RS(255,223) or RS(204,188)—and treat blind GFFT identification as advanced functionality.researching+1

Shi et al. (2023)

Paper: L. Shi, W. Zhang, Y. Chang, H. Wang, and Y. Liu, “Blind Recognition of Reed–Solomon Codes Based on Galois Field Fourier Transform and Reliability Verification,” IEEE Communications Letters, 2023.

What it solves

This work improves GFFT-based RS recognition by adding reliability verification. The motivation is that GFFT-based methods can generate incorrect candidate parameters when captured codewords are short or noisy.bohrium.dp

Main approach

The proposed process is broadly:

- Generate candidate RS hypotheses through GFFT-derived algebraic features.

- Assess how reliably each candidate explains the received data.

- Reject candidates that appear plausible algebraically but lack sufficient statistical reliability.

Why it matters

For your system, this supports an evidence-driven model. Instead of displaying:

text

RS(255,223) detected

show:

text

Candidate: RS(255,223)

Bit alignment: 3

Field: GF(2^8)

GFFT score: 0.81

Reliability score: 0.78

Decoder result: successful

Corrected symbols: 14

CRC-valid frames: 9 / 10

Confidence: high

The critical idea is that parameter identification must be confirmed by downstream decoding evidence, not only a spectral or algebraic match.bohrium.dp

4. BCH-Code Reconstruction

Reconstruction of BCH Codes Using Probability Compensation (2012)

Paper: “Reconstruction of BCH Codes Using Probability Compensation,” Asia-Pacific Conference on Communications, 2012. DOI: 10.1109/APCC.2012.6388263.

What it solves

This paper studies blind reconstruction of BCH codes from noisy intercepted streams. BCH is relevant because it is used in storage systems, satellite/telemetry links, and digital communications, and it shares cyclic-code properties with RS codes.

The target is to identify BCH-code parameters without prior knowledge of the encoder.scispace

Main approach

BCH codewords have common roots associated with the generator polynomial. The method:

- Uses cyclic-code algebraic properties.

- Tests divisibility or root-related conditions.

- Applies probability compensation to reduce false identifications caused by random bit patterns that accidentally satisfy portions of the algebraic test.

The compensation step acknowledges an important real-world issue: random or noisy data can occasionally mimic a code constraint.scispace

Why it matters

This is a good future-work reference if you want to broaden your FEC module beyond convolutional codes and RS codes.

It also teaches a general design lesson:

Do not trust one algebraic match. Estimate the probability that the match could occur by chance and require repeated, independent evidence.

Limitation

BCH blind reconstruction is not a good first implementation target. It requires finite-field arithmetic, cyclic-code theory, candidate searches, and robust validation. Add it only after convolutional and RS workflows work.

Blind Recognition of Intercepted Limited BCH and RS Codes (2025)

Paper: “Blind Recognition of Intercepted Limited BCH and RS Codes in a Noisy Environment,” 2025.

What it solves

This more recent work focuses on full-blind recognition from limited noisy intercepted data—a scenario closer to your project than methods assuming many complete codewords.

It aims to infer coding parameters for BCH and RS streams even when available capture duration is restricted and channel conditions are harsh.ieeexplore.ieee

Why it matters

This is useful if your input recordings are short bursts or partial transmissions. It can serve as a future research direction for:

- Short capture handling.

- Low-data FEC recognition.

- Noisy non-cooperative settings.

- Joint BCH/RS family discrimination.

Limitation

Because the problem is difficult, this should be considered research-grade functionality. Do not promise fully blind BCH/RS reconstruction in your GUI unless you validate it using real or synthetic captures across BER, capture length, and parameter sets.ieeexplore.ieee

5. LDPC Identification and Synchronization

Xia & Wu (2013): Nonbinary LDPC Identification

Paper: T. Xia and H.-C. Wu, “Blind Identification of Nonbinary LDPC Codes Using Average LLR of Syndrome A Posteriori Probability,” 2013.

What it solves

The paper considers blind recognition of nonbinary LDPC encoders for a predefined candidate set. This is important: it is not reconstructing any arbitrary unknown LDPC parity-check matrix from scratch.

It is designed for q-ary modulation and nonbinary LDPC systems operating over GFq.repository.lsu

Main approach

For each candidate parity-check structure, the receiver calculates syndrome-related posterior information. The candidate whose parity-check equations appear most consistent with the received soft data receives the highest score.

In simplified form:

Candidate score=1Mi=1MLLRsyndromei=0∣received data

The candidate with the largest average syndrome-validity LLR is selected.repository.lsu

Why it matters

This is a practical idea for your project:

- Maintain a library of known LDPC matrices.

- Use demodulator LLRs.

- Score each candidate Hmatrix.

- Select top candidates.

- Run min-sum or belief-propagation decoding.

- Validate using syndrome and CRC.

Limitation

It requires a known candidate set. It is not a general solution for discovering arbitrary LDPC matrices from any unknown transmission.repository.lsu

Blind Recognition of Sparse Parity-Check Matrices of LDPC Codes in Noise

Paper: “Blind Recognition of Sparse Parity-Check Matrices of Low-Density Parity-Check Codes in the Presence of Noise.”

What it solves

This research directly addresses recovery of the LDPC parity-check matrix Hfrom received codewords containing errors:

HcT=0  mod 2

The goal is to infer parity relationships despite unknown code parameters and channel noise.digital-library.theiet

Main approach

The general strategy is to search for sparse parity-check equations that are statistically consistent across multiple noisy codewords. LDPC codes are defined by sparse parity relationships, so the reconstruction method tries to recover rows of Hby finding low-weight combinations that repeatedly satisfy parity constraints.

Why it matters

This is closer to genuine blind LDPC reconstruction than candidate-library classification. It can inform a future advanced module:

text

Segment candidate LDPC blocks

→ Search for sparse parity relationships

→ Construct candidate \(H\)

→ Measure syndrome statistics

→ Run LDPC decoder

→ Validate CRC / syndrome / re-encoding

Limitation

This is computationally expensive and needs multiple aligned codewords. It is not recommended for your hackathon MVP.

Blind Frame Synchronization of LDPC Codes

Paper: “On Blind Frame Synchronization of LDPC Codes.”

What it solves

This paper studies the fact that LDPC decoding requires correct block alignment. If the receiver starts the codeword at the wrong bit position, even the right parity-check matrix fails.

It focuses on selecting the correct frame/codeword boundary using a likelihood-difference function derived from parity-check behavior.hal

Main approach

For each candidate alignment:

- Segment the stream into tentative LDPC codewords.

- Evaluate parity-check consistency under the candidate H.

- Compute a likelihood-related score.

- Select the alignment with the strongest parity-consistency evidence.

The paper reports that maximizing the likelihood-difference function gives favorable false-synchronization performance.hal

Why it matters

This suggests an important rule for your LDPC architecture:

Before declaring LDPC decoding failure, test block alignment.

For each candidate LDPC Hmatrix, search:

offset∈01…n-1

where nis the LDPC codeword length.

For each offset, calculate:

- Syndrome weight.

- Average parity-check satisfaction.

- Decoder convergence rate.

- CRC-valid frame count.

Then select the most plausible alignment.

6. Turbo-Code Reconstruction

Cluzeau & Finiasz (2009)

Paper: A. Cluzeau and M. Finiasz, “Overview of Turbo-Code Reconstruction Techniques,” 2009.

What it solves

Turbo-coded transmissions contain:

- Two or more convolutional constituent encoders.

- A turbo interleaver.

- Often puncturing.

- Potentially unknown code rate and termination.

This paper surveys techniques for blindly recovering turbo-code encoder parameters from noisy eavesdropped binary streams.eprint.iacr

Main approach

The paper discusses reconstruction techniques based on:

- Low-weight parity-check relationships.

- Constituent convolutional-code reconstruction.

- Interleaver recovery.

- Analysis of turbo-code structural redundancies.

- Noise-aware reconstruction methods.

Why it matters

This is useful background because turbo codes combine the exact three difficult topics in your problem statement:

FEC+interleaving+unknown framing/alignment

It explains why FEC and interleaver identification should often be solved jointly rather than as isolated sequential steps.eprint.iacr

Limitation

Turbo reconstruction is complex and should be a future phase only. It is not appropriate as a first decoder after Viterbi and RS support.

Turbo Convolutional-Code Parameter Estimation in Noisy, Unsynchronized Data

Paper: “Blind Parameter Estimation of Turbo Convolutional Codes: Noisy and Non-Synchronized Scenario,” 2019.

What it solves

The work addresses blind estimation of turbo-code parameters when the intercepted data is both noisy and not aligned to codeword/frame boundaries.

It includes estimation of block-interleaver parameters such as the number of rows and columns, alongside turbo-encoder properties.ntu.edu+1

Why it matters

This paper is particularly relevant if you later implement a “turbo-code mode” in your GUI. It is a good reference for handling the combined uncertainty of:

- Unknown bit offset.

- Unknown block-interleaver dimensions.

- Unknown constituent encoder parameters.

- Noisy data.

Limitation

The parameter space is large. Implementing this robustly requires substantial test data and computing time.

7. Polar-Code Recognition

Blind Identification of Polar Codes (2022)

Paper: “Blind Identification of Polar Codes Based on Estimation and Derivation Approaches,” IEEE Communications Letters, 2022.

What it solves

Polar codes are used in modern communications, notably 5G control channels. The paper proposes low-complexity methods to identify:

- Polar-code length.

- Information-bit locations.

It combines estimated parameters with parameters derived from valid shorter polar-code structures, rather than requiring a huge pre-enumerated candidate set.ieeexplore.ieee

Why it matters

This is useful if your project later expands toward cellular or 5G-related signals. It shows that different FEC families need fundamentally different recognition approaches; polar codes cannot be handled by Viterbi, RS, or standard LDPC workflows.

Limitation

Do not add polar-code detection to your current MVP unless you specifically have polar-coded recordings and a defined target standard.

8. ML-Based FEC Recognition

Blind Recognition of FEC Codes Using RNNs

Paper: “Blind Recognition of Forward Error Correction Codes Based on Recurrent Neural Network,” 2024.

What it solves

The paper proposes using an RNN-based model to recognize FEC code type and encoding parameters directly from noisy received data. It treats the problem as sequence classification rather than explicit algebraic reconstruction.bohrium.dp

Main idea

A recurrent model learns patterns across long sequences that may distinguish:

- Convolutional codes.

- Block codes.

- RS-like structures.

- Other FEC families or parameter choices.

Why it matters

This can be useful if classical methods fail because:

- Signals are noisy.

- The code family is broad.

- The bitstream is short.

- Parameters are difficult to enumerate analytically.

Major caution

An ML classifier is only as reliable as its training dataset. It may perform well on the modulation types, SNR range, code rates, interleavers, and channel conditions used in training—and fail on unseen formats.

Use ML as a candidate-ranking component, not final evidence. Confirm predictions using:

- CRC passes.

- Decoder convergence.

- Syndrome validity.

- Re-encoding checks.

- Consistent repeated frames.

Recommended FEC Research Stack

| Priority | FEC feature | Main reference | Practical output |
| --- | --- | --- | --- |
| 1 | FEC-family classification | Swaminathan et al., 2017 | Uncoded vs block vs convolutional confidence |
| 2 | Convolutional parameter estimation | Marazin–Gautier–Burel, 2011 | Candidate rate, constraint length, generator structure |
| 3 | Soft convolutional identification | Soft-GJETP, 2022 | Reliability-aware candidate ranking using LLRs |
| 4 | Soft Viterbi decoding | Standard Viterbi implementation | Decoded bits, path metric, BER estimate |
| 5 | RS known-profile decoding | Standard RS decoder plus validation | RS decode status, corrected-symbol count |
| 6 | Blind RS reconstruction | Liu–Pan–Lei, 2019; Shi et al., 2023 | Candidate RS parameters and reliability score |
| 7 | Known-H LDPC decoding | Xia & Wu candidate-set logic | Best H-matrix candidate, syndrome score |
| 8 | LDPC frame alignment | Blind LDPC frame-synchronization work | Best codeword offset |
| 9 | Sparse H-matrix reconstruction | LDPC parity-check recognition research | Candidate parity-check matrix |
| 10 | Turbo, BCH, polar, ML | Survey/research extensions | Future work |

Recommended Implementation Sequence

Phase A: Build reliable validation first

Before implementing blind FEC identification, implement:

- Bit-polarity testing.

- Differential decoding.

- Bit-order testing.

- Byte-alignment testing.

- Sync-word correlation.

- Frame-length estimation.

- CRC-8, CRC-16, and CRC-32 validation.

- A unified candidate-scoring engine.

These will validate whether a proposed FEC/interleaver configuration is genuinely correct.

Phase B: Convolutional code MVP

Implement:

- Rate-1/2 convolutional-code hypotheses.

- K=7candidate.

- NASA 17181338polynomial pair.

- Soft-decision Viterbi decoder.

- Puncturing support later.

- Re-encoding check.

- CRC validation after decoding.

This delivers the best balance of complexity, demonstrability, and relevance.

Phase C: Reed–Solomon support

Implement:

- Byte packing.

- Eight possible bit alignments.

- MSB/LSB byte-order hypotheses.

- A small library of RS profiles.

- RS decoder success/failure tracking.

- Corrected-symbol count.

- CRC/frame validation.

- Optional block-deinterleaver hypotheses.

Phase D: LDPC support

Implement LDPC only for user-selected or known H-matrices initially:

- Min-sum or normalized min-sum decoder.

- Syndrome-weight calculation.

- Iteration count and convergence reporting.

- Codeword-offset search.

- CRC validation.

Do not initially claim blind LDPC reconstruction. That requires substantially more data and computationally expensive sparse-matrix inference.digital-library.theiet+1

Evidence Model for Your GUI

For each FEC hypothesis, show actual supporting evidence:

text

Best FEC hypothesis

Family: Convolutional code

Rate: 1/2

Constraint length: K = 7

Generators: (171₈, 133₈)

Puncturing: None

Evidence

Bit offset: 3

Bit order: MSB-first

Polarity: Normal

Viterbi path metric: 0.84

Re-encoding mismatch: 1.7%

Frame sync matches: 24

CRC-16 valid frames: 23 / 24

Final confidence: 0.96

For an LDPC candidate:

text

Best FEC hypothesis

Family: LDPC

Candidate matrix: DVB-S2 short-frame rate 1/2

Codeword offset: 11

Decoder: normalized min-sum

Iterations: 18

Evidence

Initial syndrome weight: 138

Final syndrome weight: 0

Decoder convergence: yes

CRC-valid frames: 7 / 7

Final confidence: 0.98

This is more credible than simply stating that the system “detected LDPC” or “decoded RS.”