PARTIALLY Covered — Gaps You Should Address

Gap 1: Diagonal Interleaving — MISSING

The PS explicitly lists "Diagonal" as one of four required interleaver types. Your README and repo structure only implement:

- Block (matrix) ✅

- Convolutional/Forney ✅

- Diagonal ❌ — not mentioned anywhere

- Pseudo-Random ❌ — not mentioned anywhere

What's needed:

text

deinterleaving/diagonal.py     # Diagonal (helical) interleaver

deinterleaving/pseudorandom.py # Pseudo-random permutation recovery

Diagonal interleaver math (DVB-style):

text

π_diag(k) = ((k mod R) * C + (k mod R) + floor(k/R)) mod (R*C)

This is the classic DVB-T/ISDB diagonal interleaver used in terrestrial broadcast — highly relevant to the HF/VHF/UHF signals NTRO mentions.

Pseudo-random interleaver recovery is genuinely hard (it's a permutation search), but you can at least:

- Detect its presence via GF(2) rank deficiency (non-zero slope without block/Forney structure)

- Use the Burel-Gautier algorithm to recover the permutation matrix

- Support common LFSR-based PR interleavers (3GPP, DVB-RCS)

Gap 2: LDPC — MISSING

PS explicitly requires: "LDPC". Your README §9 Known Limitations admits:

"LDPC and Turbo code identification are not currently active in the blind search loop."

What's needed:

text

fec/ldpc.py    # LDPC decoder (min-sum or belief propagation)

For blind LDPC identification, the standard approach is:

- Parity-check matrix recovery via the Marazin algorithm — build a sparse binary matrix from the rank-deficient columns, then find its null space

- Codeword length detection — search for nwhere the dual code has low-density rows

- Decode using log-domain sum-product or min-sum BP

Common standards NTRO will encounter: DVB-S2 (n=64800, rates 1/4–9/10), 802.11n/ac, 5G NR LDPC (BG1/BG2).

Gap 3: Concatenated Codes — PARTIAL

PS requires: "Concatenated codes". Your pipeline handles Conv + RS sequentially (Stage B / Stage C in the joint search), but there's no explicit concatenated code decoder that:

- Recognizes the outer RS + inner Conv structure (classic DVB-S, CCSDS)

- Performs iterative turbo-like decoding between RS and Viterbi (soft-in-soft-out RS)

- Handles interleaver between inner and outer (the actual DVB-S architecture)

What's needed:

text

fec/concatenated.py   # Outer RS + inner Conv with interleaver

This is the single most important gap for real-world HF/VHF/UHF military signals, which almost always use CCSDS 131.0-B (RS(255,223) + Conv K=7 r=1/2 + interleaver).

Gap 4: Sampling Rate — WAV Metadata Trust

Your README admits:

"sample rates should be verified in the Duration dialog when not using .wav containers."

For .wav files, you do read the header sample rate. But for .iq files, you derive Fsfrom duration — which requires the user to know the capture duration. This is a manual step disguised as automation.

What's needed: Blind symbol rate + Fsestimation from cyclostationary features (you already have the cyclic autocorrelation line search in §4.4.1 — extend it to be the primary estimator, not a fine-tuner).

Gap 5: FEC Identification Confidence

Your README claims:

Post-Demod Pipeline: 54/56 (96.4%)

That's good, but the two failures are likely LDPC or concatenated cases. The Marazin estimator in fec/identify.py handles Conv + RS + Hamming but not LDPC. Adding LDPC closes this gap.

❌ COMPLETELY MISSING

| PS Requirement | Status | Impact |
| --- | --- | --- |
| Diagonal interleaving | ❌ Not implemented | High — explicitly required |
| Pseudo-random interleaving | ❌ Not implemented | High — explicitly required |
| LDPC decoding | ❌ Not implemented | High — explicitly required |
| Concatenated code decoder | ⚠️ Implicit only | Medium — needs explicit module |
| Blind Fsestimation (no duration input) | ⚠️ Partial | Medium — currently needs user hint |

3. Other Observations (Not Gaps, But Worth Noting)

3.1 Strong Points (Don't Change)

- Evidence-Based Provenance Engine (core.evidence.Parameter) — this is excellent for NTRO's "confidence" requirement

- Chiani-Martini LRT/GLRT sync search — statistically rigorous, better than naive correlation

- 220-hypothesis frame layout search — exceeds PS requirements

- Burel-Gautier GF(2) rank deficiency — correct state-of-the-art for blind de-interleaving

- Marazin blind code parameter estimation — correct approach

3.2 Minor Issues

- README.md §9 explicitly lists LDPC as a limitation — this is a direct admission of non-compliance with PS requirement (iv). Fix this before submission.

- README.md §3.3 lists interleaving as "Block and Convolutional Forney" only — diagonal and pseudo-random are absent from the supported standards table.

- deinterleaving/ directory has only gf2.py, block.py, convolutional.py — no diagonal.py, no pseudorandom.py.

4. Priority Action List

| Priority | Task | Effort | Impact |
| --- | --- | --- | --- |
| P0 | Add deinterleaving/diagonal.py (DVB-style helical) | 1–2 days | Closes explicit PS gap |
| P0 | Add deinterleaving/pseudorandom.py (Burel-Gautier permutation recovery) | 2–3 days | Closes explicit PS gap |
| P0 | Add fec/ldpc.py (min-sum BP + Marazin LDPC ID) | 3–5 days | Closes explicit PS gap |
| P1 | Add fec/concatenated.py (CCSDS/DVB-S RS+Conv+interleaver) | 2–3 days | Real-world relevance |
| P1 | Promote cyclic autocorrelation to primary Fs/symbol-rate estimator | 1–2 days | Removes manual duration input |
| P2 | Update README §3.3, §9 to reflect new capabilities | 0.5 day | Submission polish |
| P2 | Add validation tests for diagonal + PR + LDPC | 1–2 days | Evidence for judges |

5. Bottom Line

You've built ~85% of a production-grade system. The missing 15% is concentrated in exactly the areas the PS calls out by name:

"Carry out de-interleaving (Block, Convolution, Diagonal, Pseudo Random)."
"FEC (short-constrained convolution codes with Viterbi decoding, RS block codes, Concatenated codes, LDPC)."

Two of four interleaver types and one of four FEC types are absent. A judge reading your README will immediately notice §9's admission that LDPC is "not currently active." That's the single highest-leverage fix.

If you close the four P0/P1 gaps above, you'll have literal 100% coverage of every named requirement in the problem statement — which is exactly what wins these hackathons.